# TRIMBLE BUSINESS CENTER  TBC

# DATA PROCESSING OF POINT CLOUDS WORKFLOW

JOB NAME AND NO.

- Open TBC Program – SAVE OFTEN – SET UP AUTO SAVE.

- Choose New Project or open template. Template is best for settings.

- Choose Save As to save your drawing in the job folder.

- Look for .jxl ﬁle and drag into TBC project.

- Use Google Earth and pictures. Very import and a pdf of area if you have it.

- Classify points cloud – all classes.

- Extract Point Features Trees – for a large project, choose Automatic Scan; for a smaller job choose

Manual Scan.

- Extract Point Features Poles – for a large project, choose Automatic Scan; for a smaller job choose

Manual Scan – can be used for fence post change codes.

- Extract Point Features Signs – for a large project, choose Automatic Scan; for a smaller job choose Manual Scan – can be used for fence post or ﬁre hydrant change codes.

- Extract Point Features Sewer Holes – can be used for gas valves and water valves change codes.

- Select 3D View and make sure that TBC selected the correct objects.

- Extract geometry used for crosswalks – create a shape on crosswalk one square, then pick the second square and it will extract the squares. Check the ﬁrst one so it has elevations.

- Line strings – draw lines and add points use create at intervals – select two points or use distance to set feet.

- CAD Tab – Line strings used for curbs, sidewalks, joints, buildings, paint lines, or add draw lines.

- Add points using your codes and 3D view, and plan view to all your code points.

- Now that TBC has done all extractions, you can connect lines and add points.

- Use Picture station views to zoom in and out and complete the project.

- Export out custom for points ﬁle.

- Export CAD .dwg for line work set coordinate system and export as a .dgn ﬁle if needed.